## Help document for BluetoothEOG App

Software Prerequisites:

- Smartphone with minimum Android SDK 24 and Bluetooth capability

- Minimum free space of 75MB to install the app

- Arduino setup for EOG recording and wireless data transmission

- Adequate space to save the recorded EOG data files

Instructions for using the app:

- When the app is opened for the first time, a prompt for Bluetooth permissions will pop up. The user should provide Bluetooth permissions.

- Once permissions are granted, the idle app UI will be displayed which has the following:

  - Top bar with the app name- Bluetooth EOG

  - Bottom bar with the lab name- Neural Engineering Lab | IITG

  - The middle part with text: ‘Waiting for connection…’ before the app is connected to Bluetooth module attached to Arduino

  - Two spaces for Horizontal Channel and Vertical Channel EOG Graphs

  - Connect button for establishing Bluetooth connection with Arduino

  - Record button to record the data which is disabled initially before Bluetooth connection is successful

  - A data label dropdown which is disabled initially

- EOG visualization:

  - On opening the app, click ‘Connect’ button

  - On clicking, the app will try to establish Bluetooth connection with HC-05 module connected to Arduino

  - On successful connection, the app will start displaying two EOG graphs one for horizontal and one for vertical EOG respectively

  - Above the graphs, the signal amplitude will be displayed

  - Also, the Record button and Data label dropdown will be active now

- EOG Recording:

  - On opening the app, click ‘Connect’ button

  - On successful connection, the ‘Record’ button and Data label dropdown will be active

  - On Clicking the Record button, the text in the button will change to ‘Stop’ and a chronometer will appear on the left to indicate the recording duration

  - Data label can be chosen to be any of the following six: Up, Down, Left, Right, Blink and Select movement. Once recording is started, the dropdown is disabled

  - On clicking Stop button, the EOG recording is saved as a CSV file in the phone’s ‘Document’ folder in the internal storage

- Eye Movement Inference:

  - On opening the app, click ‘Connect’ button

  - On successful connection, the ‘Infer’ button will be active

  - On clicking the Infer button, the text in the button will change to ‘Stop’

  - You will be able to see the inference result beside the button on the left side

  - The inference result will be one of the following six eye movement classes: Left, Right, Up, Down, Blink and No Movement

  - On clicking Stop button, the inference operation will be terminated